User Manual

Statement

•	 This manual is the information asset owned by FF EAI Robotics

(hereafter referred to as “FF EAI Robotics”) and any reprinting of part or all

of this manual is strictly prohibited without the permission of FF EAI Robotics.

•	 This manual explains the basic components, transport and storage, specific

operations, exception handling, and technical specifications of the quadruped

robot “FX Aegis Hyper”. Be sure to read and fully understand this manual before

operating the robot.

•	 Basic information on safe use is described in detail in the "Reading Tips", so be

sure to read this part thoroughly to ensure proper use.

•	 The diagrams and photographs in this manual are representative examples and

may differ in detail from the product purchased.

•	 This manual may be modified as appropriate for product improvements,

specification changes, etc.

•	 The contents of this manual do not rule out the possibility of error or omission. If

this manual is damaged or lost or if you have questions about the contents of this

manual, please contact us promptly.

•	 Failure caused by unauthorized disassembly or modification of the product by the

customer is not covered by our warranty. See "Service and Warranty" for details.

Reading Tips

Description of Symbols

Before use (installation, transportation, maintenance, inspection), please be sure to read

and master this manual, and familiarize yourself with the equipment and safety matters

before you start using it. The safety matters in this manual are divided into "Caution",

"Mandatory" and "Prohibition". Even the contents of "Caution" may cause serious

consequences depending on the situation, so any of these safety matters are extremely

important and should be strictly observed.

Usage tips or operational recommendations. Improper using or

Caution

operating the robot may cause damage to it.

Mandatory

Matters that must be observed.

Prohibition     Matters prohibited. Misoperation is dangerous and may cause injury to

operators or damage to the robot.

Help Acquiring

For more resources to assist you in using FX Aegis Hyper proficiently, you can also visit

FF EAI Robotics’ corporate website: https://www.ff.com/

Content

1 Introduction

1

2.6 Power Off

17

2.7 Payload and Development

18

1.1 Overview

1

3 Precautions

19

1.2 Product List

1

1.3 Main Specifications

2

3.1 Work Environment

19

1.4 Power and STOP

4

3.2 Other Precautions

19

1.5 Intelligent Controller

5

4 FAQs

20

1.6 Mode and Gait

6

5 Transport and Storage

21

2 Operation

9

5.1 Transportation

21

2.1 Preparation

9

5.2 Storage

21

2.1.1 Environment

9

2.1.2 Checking

9

5.3 Carrying

21

2.2 Charging

9

6 Service and Warranty

22

2.2.1 Charging the Robot

9

2.2.2 Changing Battery

10

6.1 After-Sales Service

22

2.3 Start

11

6.2 Warranty Policy

22

2.3.1 Preparation

11

6.3 Warranty Coverage

22

2.3.2 Power on

11

6.4 Repair Instructions

23

2.3.3 Connection

11

2.4 Motion Control

12

7 Disposal

25

2.4.1 Controller Operation

12

2.4.2 Settings

15

2.5 Emergency Operation

16

2.5.1 Soft Emergency Stop

16

2.5.2 Hard Emergency Stop

16

2.5.3 Overtemperature

16

2.5.4 Falling Down

17

2.5.5 Low Battery

17

2.5.6 Other Circumstances

17

FX Aegis Hyper User Manual

1 Introduction

1.1 Overview

FX Aegis Hyper is an intelligent quadruped robot designed to meet core industry needs,

with a total of 12 degrees of freedom (DOF), capable of walking, going up and down the

stairs. The robot is equipped with a wide-angle camera, four laser radars, and two internal

hosts respectively for motion control and environmental perception calculation. An

additional host inside Intelligent Controller is used for navigation and inspections.

470mm

1130mm

285mm

470mm

1000mm

715mm

1.2 Product List

Intelligent Controller ×1

Controller ×1

Charger ×1

Repalceable Battery ×1

Aviation Plug ×8

Antistatic Cloth ×1

Robot ×1

(without battery)

Certificate ×1

User Manual ×1

Warranty Card  ×1

1

1.3 Main Specifications

3

4

5

1

6

7

2

8

9

1 Wide Angle Camera

2 Laser Radar

3 Mounting Rails

4 Intelligent Controller Power Button

5 Upper Leg

6 Power Button

7 Charging Port Cover

8 Lower Leg

9 Antislip Foot

10

11

12

13

14

15

16

10 Controller's Antenna

11 WiFi Antenna

12 Cooling Fan

13 Soft Emergency STOP

14 Hard Emergency STOP

15 Debugging Ethernet Port

16 Laser Radar

2

FX Aegis Hyper User Manual

Robot Dimensions

Sitting Size (Length × Width × Height)

1130mm×470mm×285mm

Standing Size (Length × Width × Height)

1000mm×470mm×715mm

Weight

59kg

Electric Parameters

22.4Ah(Ideal data in an environment at 25℃)

Battery Capacity

Nominal Battery Voltage

72V

Charger Input Voltage

200V~240V

Charger Output

84V/8A

Charging Time

2.5h~3h

Power Supply

12V; 24V; 5V(USB)

Communication Interface

Ethernet; WiFi; USB2.0; USB3.0

Autonomous Charging

Supported

Locomotion Parameters

Maximum Speed

4.95m/s (Data from extreme test)

Maximum Walking Speed

1.2m/s

Maximum Running Speed

2.5m/s

Maximum Slope

±30°

Maximum Step Height

20cm

Maximum Slope of Staircase

±45°

No-load Runtime

4h

Payload Runtime

2.5h

Payload

20kg (Intelligent Controller Weight Included)

Maximum Load

85kg (Data from extreme test)

3

Sensor Parameters

Laser Radar

×4

Wide-angle Camera

×1

Other Optional Modules

Bi-spectrum PTZ Camera

Environment Parameters

Ingress Protection

IP67

-20℃~55℃

Operating Temperature

※Data above is measured under ideal conditions, and the actual results may be biased.

1.4 Power and STOP

Power Button

of Intelligent

Controller

Power LED

Power Button of

Robot

Charging Port

Cover

Soft Emergency

STOP Button

Hard Emergency

STOP Button

Light OFF

Power OFF

Power LED

Light ON

Percentage & Voltage

Power ON

Light ON

Light OFF

Hard Emergency Stop OFF

Soft Emergency Stop OFF

Light ON

Light OFF

Soft Emergency Stop Triggered

Hard Emergency Stop Triggered

4

FX Aegis Hyper User Manual

Power LED

4 lights are always on

75% < Current Battery Level ≤ 100%

1 light is off, 3 lights are always on

50% < Current Battery Level ≤ 75%

2 light are off, 2 lights are always on

25% < Current Battery Level ≤ 50%

3 lights are off, 1 light is flashing

5% < Current Battery Level ≤ 25%

4 lights are off

Current Battery Level ≤ 5%

4 lights cycle on

Charging: Current Battery Level ≤ 25%

1 light is always on, 3 lights cycle on

Charging: 25% < Current Battery Level ≤ 50%

2 lights are always on, 2 lights cycle on

Charging: 50% < Current Battery Level ≤ 75%

3 lights are always on, 1 light flashes

Charging: 75% < Current Battery Level ≤ 100%

1.5 Intelligent Controller

Indicator light

Intelligent Controller's

Power Button

Light OFF

Intelligent Controller OFF

Light ON

Intelligent Controller ON

Intelligent Controller Indicator Light

Intelligent Controller is working normally.

Always on and green light

Green flashing light

Robot is charging.

Yellow flashing light

Robot is avoiding obstances when in navigation.

Red flashing light

Robot battery is not enough.

Light off

Intelligent Controller is not powered on.

5

5

7

8

9

6

1

2

3

4

⑤USB2.0×2

Interface

①LAN1+12V

⑥ LAN3+12V

②LAN2

⑦ LAN4+12V

③24V

⑧ LAN5+24V

④USB3.0x2

⑨ LAN6+24V

※When the USB 3.0 interface is not in use, the intelligent controller maintains an IP67 protection rating.

If the dust- and water-proof cap is removed to use the USB 3.0 intelligent, additional protective measures

are required to preserve dust and water resistance.

1.6 Mode and Gait

Control Mode

Manual

Robot is completely controlled by the user through the controller

The perception host will monitor the surrounding environment and

Assist

assist the user in controlling the robot

Navigation

Robot moves autonomously and can not be controlled with the controller

Body Height

Normal

Robot stands normally

Crawling

Suitable for passing through the terrain with smaller longitudinal height

6

FX Aegis Hyper User Manual

Gait

Walking

Suitable for flat surfaces such as concrete (limited max speed: 1.2m/s)

Running

Suitable for flat surfaces such as concrete (limited max speed: 2.5m/s)

Slope

Suitable for slopes less than 30° or other gentle irregular terrain

Suitable for complex terrain through irregular obstacles such as stone

Off-Road

piles and ruins

It can perceive the shape of the staircase and calculate the foothold on

Stair

its own, which is suitable for stairs with a slope under 45°

It can perceive the shape of the staircase and calculate the foothold

45° Stairs

on its own, which is suitable for stairs with a slope of 45°, and is only

supported in multi-frame mode

Utilizes AI algorithms, suitable for surfaces with slight undulations

L-Walk

under normal conditions

Utilizes AI algorithms, suitable for complex terrain with height

Mountain

differences, slopes and stairs

Utilizes AI algorithms, suitable for scenarios requiring reduced robotic

Silent

motion noise.

※L-Walk is suitable for surfaces with little undulations, including but not limited to flat surfaces, muddy

surfaces, flooded surfaces, undulating surfaces with small differences in height, stairs with a single step of

15cm, slopes of 18° , etc.

※Mountain is suitable for surfaces with large undulations under normal conditions, including but not

limited to rubble surfaces, bumpy surfaces with large differences in elevation, stairs with a single step of

25cm, and slopes of 30° .

※Silent is suitable for noise cancellation demand scenarios, the gait noise when standing still

≤ 50dB,

walking noise ≤ 60dB, does not support walking up and down stairs, walking on slopes.

•	 When the robot passes through stairs or slopes, do not stand on the

stairways, platforms or slopes below the robot, to avoid possible

personal injury if the robot falls.

•	 The robot can only climb stairs when its front faces the upward

direction of the staircase.

Using mountain gait generates a lot of heat, and the continuous use

of this gait to cross obstacles within a short period of time will easily

cause the robot to overheat, so it is recommended not to use this gait to

climb stairs for more than 8 minutes continuously.

7

Elevation Map Mode

Suitable for scenarios where the robot's landing plane

Solid ground mode

is solid, such as a normal solid concrete flat floor or

concrete stairs.

Suitable for scenarios where the robot's landing plane

is a grid-like pattern with gaps in between, such as an

Grid ground mode

industrial platform or staircase made of steel grating

panels.

A special condition of solid ground, which refers to a

Stair without risers mode

staircase where the robot places its feet on a solid surface

without risers.

Suitable for scenarios including multiple types of ground

Multi-frame mode

surfaces or 45° stairs. For example, if the robot moves

from solid ground to grid ground.

8

FX Aegis Hyper User Manual

2 Operation

2.1 Preparation

2.1.1 Environment

•	 Please ensure that operators and non-operators present have

read the manual carefully and understand the basic operating

instructions and safety precautions.

•	 Before start FX Aegis Hyper, ensure that all people or objects

present are more than 2 meters away from the robot to avoid

collisions.

•	 Please use FX Aegis Hyper in an environment of -20℃~55℃.

2.1.2 Checking

•	 Check the battery indicator. It is suggested to use the robot when

the battery is at least 75%.

•	 Make sure that the emergency stop buttons stay off.

•	 Make sure there is no visible damage to the exterior of the robot.

•	 Make sure the joystick used for remote control is fully charged.

If the robot parts are aging or damaged, please do not start the robot

and contact the after-sales staff in time.

2.2 Charging

2.2.1 Charging the Robot

FX Aegis Hyper is powered by a ternary lithium battery.

You can connect the robot with 220V AC power supply through the charger, and the charging

port is located on the left side.

1. First make sure the robot is turned off. Open the charging port cover and connect the

charger to the charging port and tighten it (charger has an anti-reverse plug-in port), then

connect it to 220V AC power.

2. During charging, the charging indicator on the charger is red.

3. When charging is complete, the charging indicator on the charger turns green.

9

2.2.2 Changing Battery

The battery of FX Aegis Hyper is replaceable, so users can take out the battery and replace

it with other batteries, and charge the removed battery separately.

1. Unscrew the two hand-screws, remove the cover plate, and unplug the connector:

Battery Handle

Remove

Charging Port

hand-screw

hand-screw

Battery Connector

2. Straighten the battery handle outward and take out the battery, then connect the charger

for charging:

Straighten the

Remove the battery

handle

and charge it

3. When charging normally, the charging indicator light on the charger is red; when

charging is complete, the charging light on the charger turns green.

•	 It is recommended to charge in an environment of 0 ℃ to 45℃.

•	 During charging, please do not power on the robot and always

pay attention to the state of the charger during charging to avoid

accidents. Disconnect the charging power supply in time after

charging.

•	 When the robot is not charging, the charging port cover must be

closed to prevent water.

10

FX Aegis Hyper User Manual

2.3 Start

2.3.1 Preparation

1. Take the robot from the transport case and place it on a flat surface, as described in the

section "Transport and Storage".

2. Adjust the robot pose as required (as shown in the picture above): knee joints retracted,

hip front swing joints swung back, side swing joints vertical, and the bottom of the body

touching the ground.

2.3.2 Power on

Press the power button to turn on the robot. At this time, the power button is always on. At

the same time, it is necessary to ensure that the light of soft emergency stop is off and the

light of hard emergency stop is on. If not, press the hard emergency stop button or rotate

the soft emergency stop button in the direction indicated by the arrow to turn it off. If the

hard emergency stop is triggered, it is necessary to restart the robot after turning off the

hard emergency stop.

2.3.3 Connection

Long press the power button to turn on the controller (please ensure that the controller is

fully charged). The robot and controller have been paired and bound. After opening the app,

the controller will automatically connect to the robot it is bound to.

11

2.4 Motion Control

2.4.1 Controller Operation

2

3

12 13 14

11

15

17

16

18

24

19

23

20

21

22

5

6

7

8

9

10

4

1

25

28

26

27

12

Flip to the center simultaneously

Cancel Soft Emergency Stop

FX Aegis Hyper User Manual

Button

①Controller Power

Press and hold to power on/off, and press once to wake

Flip up simultaneously

Trigger Soft Emergency Stop

②SW1

③SW4

④Start/Stop Moving

Start or stop moving, that is, to start or stop stepping

Stand up or sit down

⑤A

Switch to Torque Control Mode to unlock each joint

⑥B

Switch to Walk gait

⑦C

Switch to Run gait

⑧D

Switch to Slope gait

⑨E

Switch to Off-Road gait

⑩F

View APP version/Set language/Jueying Lab/User Protocal/Save

⑪Settings

data

⑫Buttons Introduction Show the introduction to physical buttons

⑬Screenshot

Save images for the video streaming

⑭Record

Record a video of the video streaming

Displays the connection status：

connected

/connecting

/

⑮Robot Signal

disconnected

⑯Robot Battery

Real-time display of robot signal strength

⑰Controller Battery

Real-time display of the controller's remaining battery

Soft emergency stop:Forcing the robot to lie down, which is

⑱STOP

generally used when the robot dog is out of control or in an

emergency

Select the elevation map mode, from top to bottom: Solid

⑲Elevation Map Mode

Ground/Grid Ground/Stair without Risers/Multi-Frame

Select body height: Normal/Crawling,and the button displays the

⑳Body height

current status

㉑Stand/Sit

Stand up or sit down and the button displays the current status

㉒Torque Control

Switch to Torque Control Mode to unlock each joint

㉓Control Mode

Select the control mode:Manual Mode/Asist Mode/Nav Mode

13

Button

To enable the robot dog to perform autonomous charging, please

㉔Auto Charging

refer to the autonomous charging manual for specific use

Show the real-time video

Flip to the center

from the robot

㉕SW2

Close the real-time video

Flip up or down

from the robot

㉖Gait

Select gait, which can be switched only in the stepping status

㉗Start/Stop Moving

Start or stop moving, that is, to start or stop stepping

㉘2D obstacle point

Open the Jueying Lab to show a top-down 2D obstacle point

clouds

cloud

•	 After connecting the robot, ⑮will display as connected, at which point you can start

controlling the robot.

•	 After standing up, each joint of the robot is in a protective locking state, and in this state,

it can switch between lying down and standing by ⑤ or ㉑, or switch to force control

mode by ⑥or ㉒, and the locking state of each joint can be lifted, so that it can move in

steps.

•	 To make the robot walk, first make the robot stand up, then press ⑥or ㉒ to switch to

torque control mode, and then press ④or ㉗ to make the robot start moving, and the

robot will start stepping. At this time, the left joystick can control the robot to move

forward, backward, left and right, the robot will adjust the movement speed according to

the amplitude of the joystick, and after releasing the joystick, the robot will slow down to

zero and step in place; The right joystick controls the robot to turn left and right. Press

④or ㉗  again to end walking, and if the robot is walking forward, it will take a certain

amount of time to slow down and stop.

•	 Soft emergency stop on Controller: Triggered by flip ② and ③up simultaneously or

clicking ⑱.

•	 Through ⑲, the stair gait can be assisted to carry out foothold planning in manual

mode, and the correct elevation map mode needs to be selected according to the type of

staircase ground when used; In the assist mode, it can assist the user to avoid obstacles,

and it is necessary to select the correct elevation map mode according to the ground in

the actual environment to make the obstacle avoidance function work normally.

•	 Multi-frame mode can only be enabled when the robot is standing still.

14

FX Aegis Hyper User Manual

2.4.2 Settings

29

30

31

32

33

Button

㉙APP Version

View the current APP version

㉚Language

Select language: 中文 /English

㉜User Agreement

Click "View" to view the user agreement

㉝SAVE DATA

Save the data of robot operation

When the robot is abnormal, you can use the ㉝SAVE DATA to record the abnormal data

for troubleshooting. Please ensure that the robot dog is in the state of getting down or

emergency stop before using it.

15

FX Aegis Hyper User Manual

2.5 Emergency Operation

2.5.1 Soft Emergency Stop

If the robot's legs swing or shake violently and other abnormal phenomena occur during

use, please start the soft emergency stop function on Controller (flip ②and ③up at the

same time or click the button ⑱) or push the soft emergency stop button at the tail of

the robot to make the robot lie down and enter the self-locking protection state. If the

soft emergency stop is triggered by ②and ③ , please flip them back to release the soft

emergency stop. If the rear soft emergency stop button is pressed, you need to rotate

the  button in the direction of the arrow on the button to release it. After releasing the

soft emergency stop, please release the force in time (press ⑤or ㉑) to release the self-

locking protection. After troubleshooting, press the stand button again to operate the robot

normally.

2.5.2 Hard Emergency Stop

Hard emergency stop, that is, hardware emergency shutdown, press the hard emergency

stop button at the tail of the robot, and the indicator light is off to indicate that the hard

emergency stop has been triggered, which can ensure that the robot joints are locked in

the boot state and avoid the robot from getting out of control. At this time, press the hard

emergency stop button again, and the indicator light will be on to indicate that the hard

emergency stop has been lifted, and the robot needs to be restarted to restore the normal

use state after the hard emergency stop is lifted.

Once the hard emergency stop is triggered, it will cause the robot to lose

all kinetic energy and thus fall to the ground. There is a risk of damaging

the ground or the robot, so it is strictly forbidden to press the hard stop

emergency button during normal movement!

2.5.3 Overtemperature

The system comes with temperature sensing. Once the robot runs for a long time causing

the motor or drive to overheat, it will automatically start the overtemperature protection

and the robot will automatically stop moving and get down in place.

16

2.5.4 Falling Down

If the robot falls down suddenly, the joints will be automatically locked. Please try to click

[Stand] button in the app after making sure there is no obstacle around. If the robot still

can't get up, press the emergency stop button at the rear of robot, then click [Save Data] at

the Settings page of the APP and power off the robot.

2.5.5 Low Battery

When the robot's battery is below 10%, it will enter a low battery warning state, and it

should be charged immediately. When the battery level drops below 3%, the robot will

trigger a low battery protection mode, where it will automatically lie down and will no

longer respond to commands from the remote controller.

2.5.6 Other Circumstances

•	 If the joints are not locked or are still swinging after the robot falls down suddenly, wait

30 seconds after the robot joints have completely stopped moving before pressing the

hard emergency stop button at the rear of the body.

•	 In case of a fire, do not use water to extinguish it. Please use foam fire extinguisher, dry

powder fire extinguisher or carbon dioxide fire extinguisher nearby.

•	 If the soft emergency stop button fails or smoke from or water in robot or other

unexpected situations, please immediately power off the robot and wait until it's

safe to identify the problem. Then please contact FF EAI Robotics, and we will help to

troubleshoot the problem and repair or change your robot. Pay attention to safety in use!

•	 If the robot falls, do not drag, push or flip it before triggering the hard emergency stop.

2.6 Power Off

Make sure that the robot is sitting before the following operations.

Press the power button to power off the robot.

After powering off, please cover the robot with the antistatic cloth to

avoid dust contamination of laser radar and other devices.

17

FX Aegis Hyper User Manual

2.7 Payload and Development

Users can attach payloads to the back of the robot or develop new functions based

on software interfaces. To learn more about payload development, usage of software

interfaces and packages, refer to Application Manual and API Documentation.

•	 After attaching payloads to the back of the robot, please modify

the parameters of payload. To learn more about how to modify the

parameters, refer to the chapter "2.2 Paramters Configuration" in

Application Manual, or contact after-sales staff.

•	 The maximum power supplied by the robot to the payload is 600W.

For details about the maximum power provided by each power port,

read the chapter "5.2 Hardware interface and wiring definition" in

Application Manual.

•	 Please cover the aviation plug with its cover.

18

3 Precautions

3.1 Work Environment

•	 Please do not operate the robot in environments with strong WiFi

signal interference. Be sure to turn off part or all other WiFi signal

sources, and then use the joystick to operate the robot.

•	 Do not operate the robot in such bad weather as fog, snowing,

lightning, sandstorms, windstorms, tornadoes, etc.

•	 Keep the robot in sight and always keep it at least 2 meters away from

people, water, open flames, etc.

•	 When using the robot on smooth surfaces such as glass and tiles,

avoid violent movements and apply Stair or Slope gait to prevent the

robot from slipping and falling down.

•	 Do not operate the robot on the edge of a high place to prevent it from

falling and causing damage.

•	 Please consult the after-sales staff first before operating the robot in

environments with strong electromagnetic interference.

3.2 Other Precautions

•	 When carrying the robot, pay attention to the anti-pinch label on the

robot and do not put your hands into the position where the anti-pinch

label is attached!

•	 Do not lift the robot while it is moving to avoid unintended movements

that could cause damage to robot or injury to people!

•	 Please do not download other unrelated APPs in the controller.

•	 Pay attention to the sealing label on the robot. It is strictly forbidden

to disassemble the robot personally. Once disassembled, the warranty

will be invalid!

19

of the APP to save the abnormal motion data, and then restart the robot. If it does not

FX Aegis Hyper User Manual

4 FAQs

Q1: Is it normal for a robot to stop moving on its own?

A:  The motor or driver may be overtemperature protected. Please wait 10 minutes and

try again. If you still can't control the robot to move, please check if the battery is fully

charged, if the hard emergency stop button is turned off (the button lights green).

Q2: What if there is no response to the commands from APP after connected?

A: First confirm if the hard emergency stop is off (the button lights green). If the APP is still

unable to control the robot, please restart the robot.

Q3: Can the robot continue to be used when its body or legs are askew after standing up?

A: At this time, do not continue to click [Torque Control]. Please immediately trigger soft

emergency stop to make the robot lie down. Then shut down and restart to try to control

the robot to stand up again. If still abnormal, please contact the after-sales staff.

Q4: What terrain can the robot adapt to?

A: The robot can be applied to gently uneven ground such as flat land and grass, and can

also climb complex terrain such as stairs and slopes, but please avoid operating and using

the robot in extreme terrain (such as too smooth surfaces such as ice, sharp or undulating

terrain, near high edges, etc.).

Q5: What should I do if the robot suddenly falls, stumbles, or has an abnormal gait

without being disturbed by external forces?

A: Please press the soft emergency stop in time, and click [Save Data] at the Settings page

return to normal after restarting, please contact after-sales service for help.

Q6: What if encountering a problem that cannot be solved even after consulting this

manual?

A: Click [Save Data] at the Settings page of the APP and contact the after-sales staff in

time.

20

5 Transport and Storage

5.1 Transportation

The transport case is 1.09m×0.55m×0.57m.

•	 Before the transport case is being transported on the truck, make

sure to step on the two diagonal brake wheels on the transport box.

•	 During shipping the robot with transport case, make sure the front of

the transport case is facing up.

5.2 Storage

•	 FX Aegis Hyper shall be stored in a clean and dry place at -20℃~55℃.

•	 Robot must be powered off.

•	 Do not pour water or other liquids on the robot.

•	 Do not place other objects within the joint rotation range.

•	 Cover the robot completely with a clean antistatic cloth after use.

•	 It is recommended to store FX Aegis Hyper in the specifically designed transport case

to protect it from shock and vibration.

•	 FX Aegis Hyper must be placed in the transport case with its back facing up.

•	 If the robot is not used for a long time, charge it every three months to keep the battery

active.

5.3 Carrying

FX Aegis Hyper weights approximately 59 kg and it is recommended that two people carry

it: one person lifts the anterior carrying point of the robot, while the other lifts the rear

carrying point of the robot.

Carrying Point

Carrying Point

•	 Power off FX Aegis Hyper and carry it gently.

•	 Do not hold joints to prevent pinching or even scratching when carrying.

•	 The metal components of the robot's legs that have just finished running

are hot, so please wait until it cools down to suitable temperature before

carrying.

21

FX Aegis Hyper User Manual

6 Service and Warranty

6.1 After-Sales Service

•	 After the delivery of the equipment, FF EAI Robotics will send the specialized staff to

check the equipment and install the system as agreed.

•	 Provide free user training to guide users to operate the equipment, and arrange

technical support staff to provide timely after-sales service to ensure the smooth

operation of the system.

6.2 Warranty Policy

The warranty period for the major components of FX Aegis Hyper is as below.

Component

Warranty Period

Joint Module, Upper Legs, Lower Legs

6 months

Battery, Laser Radar, Wide-angle Cameras, Hosts

12 months

Tip: Shell, foot, other wear components, transport case and other accessories are not covered by warranty.

If necessary, please consult the after-sales staff.

The warranty period starts from the acceptance date. Products or components that are in

the warranty period and covered by warranty acquire free warranty service. If the product

you purchased is beyond the warranty period, you can also be helped by us through

purchasing the separate service.

6.3 Warranty Coverage

Depending on the specific situations, we will repair or replace components for the product

you purchased. However, the following situations will not be covered by the free warranty,

but you can still choose the paid warranty service. Please consult the after-sales staff for

details:

•	 Damage caused by man-made non-product quality problems.

•	 Unauthorized modification, disassembly, shell opening, etc.

•	 Out of warranty.

•	 Damage caused by improper installation, use and operation (such as dropping, crushing,

immersion, violent use, etc.) as required by the manual.

•	 Damage caused by secondary development.

•	 Damage caused by use in excess of the safe weight capacity.

•	 Damage caused by self-repair or replacement of parts without official instructions.

•	 Damage caused by installing third-party products by yourself.

•	 Damage caused by the use of a non-original battery pack for power supply.

22

•	 Failure or damage due to force majeure factors such as typhoon, earthquake, fire,

lightning strike, abnormal voltage.

•	 Damage caused by operating the robot dog in an environment with severe signal

interference.

•	 Damage caused by improper handling of the robot on slippery surfaces such as glass

and ice.

•	 Fallen damage caused by the use of a robot near a high edge.

•	 Fallen damage other than operate the robot on flat terrain.

•	 Damage caused by operation in environments such as sharp terrain and large

undulations.

•	 Damage caused by forced operation of parts in the event of aging or damage.

•	 Damage caused by not keeping a safe distance when used in complex environments.

•	 There are obvious traces of dust, grit, water, and metal powder intrusion inside the

robot.

•	 Damage caused by improper handling operations.

•	 Damage caused by violent or non-violent means applied to the robot when it exceeds its

anti-interference limit.

•	 Due to operation errors, there are obvious marks of collision and scratches on the

surface of the robot.

6.4 Repair Instructions

•	 Before acquiring after-sales service, please make sure to back up all data and delete

important data to prevent data loss or leakage. FF EAI Robotics is not responsible for any

data loss or leakage in the product.

•	 When you acquire after-sales service from FF EAI Robotics, you authorize FF EAI Robotics

to make any modification, delete data or restore factory settings for the purpose of after-

sales service.

•	 Before sending it for repair, please contact the after-sales staff and FF EAI Robotics will

try to diagnose and solve your problem remotely.

•	 If the above methods cannot solve your problem, you can send the robot for repair after

verifying with the after-sales staff. You need to pay for the postage when you send the

product to FF EAI Robotics. After FF EAI Robotics receives the problematic product in

need of repair, the product will be tested to determine the problem and responsibility.

•	 If the problem is caused by defects in quality of the product itself, FF EAI Robotics will be

responsible for the testing fee, material fee, labor fee and your postage.

•	 If the product does not meet the conditions of free repair after testing, you can choose

to pay for repair, and the testing fee, material fee, labor fee as well as the postage for

sending back shall be paid by you. You can also choose not to repair and to send product

back to you, and the postage and insurance fee shall be paid by you.

•	 Considering environmental protection and safety, please do not send seriously damaged

23

FX Aegis Hyper User Manual

batteries. If you have sent, FF EAI Robotics will scrap such batteries and will not return

them back to you.

•	 If you provide an incorrect delivery address which results in non-delivery or rejection by

the recipient, the adverse consequences and losses shall be borne by you.

•	 To ensure your rights and interests, when you sign for the after-sale product sent by

FF EAI Robotics, please check carefully whether the product is intact. If there is any

abnormality, please immediately take video or photos on the spot and contact FF EAI Robotics

to get the solutions. If there are unresolved after-sale problems, please also

contact FF EAI Robotics immediately, otherwise it is deemed as the end of this after-sale

service without dispute.

※FF EAI Robotics reserves the right of final explanation of the after-sales terms.

24

Thank you for choosing FX Aegis Hyper.

If a non-user-induced quality issue arises during the applicable warranty period, please contact FF after-sales support with your proof of purchase and product serial number. We will assist with remote diagnostics and, based on the assessment results, provide repair or replacement support in accordance with the FF AI Robotics General After-Sales Service Policy.

Returns and exchanges are handled through the original point of purchase and in accordance with applicable local consumer protection laws. Complete terms are governed by the FF AI Robotics General After-Sales Service Policy.

After-sales support: support.ff.com

After-sales email: support@ff.com

7 Disposal

•	 The disposal of waste robots and components shall be carried out in accordance with

the corresponding national laws and regulations on the recycling of wasted electric

appliance and electronic products.

•	 Especially the use or disposal of lithium batteries contained in robots shall be subject to

national laws and regulations regarding the disposal of batteries.

25