Overview

Master Mini is a humanoid robot development platform for competitions, education, entertainment, and related scenarios. It is affordable, lightweight, and durable.

1.1 Product Composition

The MASTER MINI robot consists of the head, torso, arms, and legs. It has 22 degrees of freedom across the whole body, enabling flexible motion and posture control.

- The head has 2 degrees of freedom, including the head yaw joint and head pitch joint. The depth camera and microphone array are located in the head.

- Each arm has 4 degrees of freedom, including the shoulder pitch joint, shoulder roll joint, shoulder yaw joint, and elbow joint.

- Each leg has 6 degrees of freedom, including the hip pitch joint, hip roll joint, hip yaw joint, knee joint, upper ankle joint, and lower ankle joint.

- The torso contains the controller, speaker, and battery.

See details in Specifications

1.2 What Can It Do?

Motion Capabilities. MASTER MINI supports omnidirectional walking and compound gaits, including forward/backward, lateral, and rotational movement. It can walk on uneven ground, has strong disturbance resistance, and actively recovers balance when subjected to a certain external force.

Built-in Agents. MASTER MINI includes 6 official Agents, which work like ready-to-use "apps" covering entertainment, conversation, teaching, competition, and other scenarios. Developers can also create their own Agents and share them with other users.

| Agent | Scenario | Core Capability |
| --- | --- | --- |
| Master Mini | Getting Started | A general-purpose basic Agent that supports daily actions such as waving, handshaking, and bowing with folded hands. It can also perform several fun dances for everyday interaction scenarios. |
| Soccer | Football Competition | Designed for soccer scenarios. It can autonomously track, chase, kick, and shoot the ball, and can accurately reproduce signature celebration moves from soccer stars such as Cristiano Ronaldo and Messi. |
| Hi Chat | Voice Interaction | An intelligent conversational Agent that can switch flexibly among fun personas such as a cute child, tutor, or pet, meeting needs for casual chat and entertaining companionship. See details in Voice Interaction |
| Teach | Teaching | A hands-on learning Agent. Users physically guide the robot through a motion, and the device quickly recognizes and accurately reproduces it, making custom actions easy to create. |
| Dance | Entertainment | Specialized in a wide range of Chinese and Western dances, including folk, trendy, and classic dance moves, while also mastering advanced actions such as roundhouse kicks. |
| Lion Dance | Festival Performance | Focused on traditional folk performance, skillfully performing authentic Southern-style lion dance and showcasing the charm of traditional Chinese culture. |

Safety Protection. When the robot enters an uncontrollable state, it automatically switches to DAMP mode to prevent damage. In dangerous situations, the joystick, App, and back-panel buttons can all trigger a soft emergency stop.

2. Specifications

2.1 Product Specifications

| Type | Specification Parameters | Master Mini | Master Mini Pro | Master Mini Ultra |
| --- | --- | --- | --- | --- |
| Basic Parameters | Height (when standing upright) | 0.95m | 0.95m | 0.95m |
|  | Weight | About 19.5kg | About 19.5kg | About 19.5kg |
| DoFs | Total DoFs | 22 | 22 | 22 |
|  | Single Leg DoFs | 6 | 6 | 6 |
|  | Single Arm DoFs | 4 | 4 | 4 |
|  | Head DoFs | 2 | 2 | 2 |
| Operational Parameters | Walking Speed | 1.1m/s | 1.1m/s | 1.1m/s |
|  | Turning Speed | 1.5rad/s | 1.5rad/s | 1.5rad/s |
| Battery Parameters | Battery Capacity | 2Ah | 5Ah | 5Ah |
|  | Battery Life | 20min（1.1m/s） | 1h10min（1.1m/s） | 1h10min（1.1m/s） |
|  | Charging Time | ≤2h | ≤2h | ≤2h |
|  | Cycle Life | ≥500 times | ≥500 times | ≥500 times |
| Computing Platform | Computing | 48 TOPS (Dense) AI performance | 117 TOPS AI performance | 200 TOPS AI performance |
| Sensors | Camera | Depth Camera | Depth Camera | Depth Camera |
|  | Microphone | Microphone Array | Microphone Array | Microphone Array |
|  | Speaker | 1 | 1 | 1 |
| Interaction interface | Buttons | Power×1, Interaction*3 | Power×1, Interaction*3 | Power×1, Interaction*3 |
|  | Indicator Light | RGB LED*1 | RGB LED*1 | RGB LED*1 |
| Safety Fuction | Auditory Alerts | Low Battery Alert, Joint Overheat Alert | Low Battery Alert, Joint Overheat Alert | Low Battery Alert, Joint Overheat Alert |
| Communication Methods | Wired connection | Gigabit Ethernet | Gigabit Ethernet | Gigabit Ethernet |
|  | Wireless Connection | WIFI 6 | WIFI 6 | WIFI 6 |
|  | Bluetooth | Bluetooth 5.2 | Bluetooth 5.2 | Bluetooth 5.2 |
|  | Certification Standards | CE/FCC | CE/FCC | CE/FCC |
| Noise | Walking Noise | ≤70dB | ≤70dB | ≤70dB |
| Environmental Adaptability | Environmental Adaptability | -10℃~45℃ | -10℃~45℃ | -10℃~45℃ |
|  | Operating Humidity | 5%~90%，without condensation | 5%~90%，without condensation | 5%~90%，without condensation |

2. 2 Main Components

2.2.1 Joint Motors

2.2.1.1Joint IDs and Limits

| Joint ID | Joint Name | Limit (Degrees) |  |
| --- | --- | --- | --- |
|  |  | Max | Min |
| 0 | Head Yaw Joint | 59 | -59 |
| 1 | Head Pitch Joint | 49 | -19 |
| 2 | Left Shoulder Pitch Joint | 69 | -169 |
| 3 | Left Shoulder Roll Joint | 94 | -94 |
| 4 | Left Shoulder Yaw Joint | 109 | -109 |
| 5 | Left Elbow Joint | 39 | -129 |
| 6 | Right Shoulder Pitch Joint | 69 | -169 |
| 7 | Right Shoulder Roll Joint | 94 | -94 |
| 8 | Right Shoulder Yaw Joint | 109 | -109 |
| 9 | Right Elbow Joint | 129 | -39 |
| 10 | Left Hip Pitch Joint | 128 | -170 |
| 11 | Left Hip Roll Joint | 89 | -22 |
| 12 | Left Hip Yaw Joint | 59 | -59 |
| 13 | Left Knee Joint | 133 | 0 |
| 14 | Left Ankle Up Joint | 38 | -17 |
| 15 | Left Ankle Down Joint | 41 | -16 |
| 16 | Right Hip Pitch Joint | 128 | -170 |
| 17 | Right Hip Roll Joint | 22 | -89 |
| 18 | Right Hip Yaw Joint | 59 | -59 |
| 19 | Right Knee Joint | 133 | 0 |
| 20 | Right Ankle Up Joint | 38 | -17 |
| 21 | Right Ankle Down Joint | 41 | -16 |

2.2.1.2 Coordinate Systems

When all joints are at zero degrees, the coordinate systems of the joints are shown below.

2.2.2 Controller

|  | Master Mini | Master Mini Pro | Master Mini Ultra |
| --- | --- | --- | --- |
| Processor | High-performance ARM Processor | Jetson Orin NX 8GB | Jetson AGX Orin 32GB |
| Computing performance | 1×GoldP@3.2GHz 4×Gold@2.8GHz 3×Silver@2.0GHz 48 TOPS (Dense) | 6-core Cortex-A78AE CPU@2GHz AI performance: 117 TOPS | 8-core Cortex-A78AE CPU@2.2GHz AI performance: 200 TOPS |
| Memory | 8GB | 8GB | 32GB |
| Storage | 128GB | 512GB | 512GB |
| Wired Network | 1000M*1 | 1000M*1 | 1000M*1 |
| Wireless Network | Wi-Fi 6*1 | WIFI6*1 | Wi-Fi 6*1 |
| Audio | Microphone, speaker | Microphone, speaker | Microphone, speaker |

- Safety and Warnings

Whenever the robot moves, stands up, or performs actions, there is a risk of falling, collision, or personal injury. The following items are the most common real-world risk points. Read them before operating the robot for the first time.

❗Read Before Operation

- Before entering WALK mode, always enter PREP mode first and make sure the robot is standing securely on the ground.

- Do not lift the robot while it is moving.

- During motion, make sure there are no people or obstacles within the robot's operating range to avoid injury or collision.

- During motion, do not touch any part of the robot with your body except the handle.

3.1 Venue and Actions

Large full-body actions such as full-body dances should be performed on hard surfaces such as rubber, cement, or tile. Carpet has high resistance and can increase the risk of falling. Keep people away from the robot throughout the action. For detailed venue and environment requirements, see "Overview".

3.2 During Upgrade

When installing new software, the robot's motion-control program is stopped and the joints no longer output torque. Before upgrading, make sure the robot is well supported, for example lying flat on the ground, and then switch it to DAMP mode to prevent it from falling during the upgrade.

3.3 Additional Notes for Secondary Development

After entering CUSTOM mode, all joints, including the legs, are controlled by your code, and the robot can easily lose balance and fall. Use a hoist to protect the robot throughout the process, and validate algorithms in Webots / Isaac simulation before running them on the physical robot. Be especially careful with changes involving biped control. CUSTOM mode can only be entered from PREP mode or DAMP mode, and can only switch back to those two modes. For the full mode description, see "Modes"。

3.4 FCC Compliance Statement

Product Name: Master Mini

Model: Master Mini

This device complies with Part 15 of the FCC Rules. Operation is subject to the following two conditions:

- this device may not cause harmful interference, and

- this device must accept any interference received, including interference that may cause undesired operation.

Responsible Party: AlphaBeta Tech LLC

Address: 3080 OLCOTT ST UNIT B108 SANTA CLARA, CA 95054

Contact: alphabetallc@outlook.com

- Modes

- The robot has multiple mode

- Different mode determines which action are available

- Different mode can switch between one another, but with certain constraints. For example, DAMP mode cannot switch directly to WALK mode; it must first enter PREP mode

4.1 State map

4.2 DAMP Mode (DAMP)

- The robot is powered on and the main controller is running normally.

- All joints enter a damped state. That is, when their positions are changed, there is resistance resistance; the joints do not actively change position and do not try to hold position.

- In DAMP mode, the robot cannot stand and needs effective support.

- DAMP mode is a safe mode and is less likely to cause accidental injury.

- DAMP mode can switch to PREP mode , but cannot switch directly to WALK mode

4.3 PREP Mode (PREP)

- The robot is powered on and the main controller is running normally.

- The robot enters and maintains a standing posture. That is, the robot assumes and holds a standing pose. If you try to change a joint position at this time, there is very strong resistance , and after displacement the joint will actively recover to return to the standing posture.

- In PREP mode, the robot can stand steadily and remain balanced on the ground. However, it cannot withstand large disturbances in this state because it only holds its posture and does not try to recover balance.

- PREP mode can switch to any other mode, including DAMP Mode、WALK mode

4.4 WALK Mode

- The robot is powered on and the main controller is running normally.

- The joints can execute corresponding preset action , including omnidirectional walking, turning, stepping, standing still, and head turning.

- Compared with PREP mode, WALK mode has stronger disturbance resistance and actively tries to recover balance after external forces within a certain range.

- WALK mode can switch to any other mode, including DAMP mode and PREP mode

- Before entering this mode, make sure the robot is already in PREP mode and standing on flat ground.

4.5 CUSTOM Mode

- The robot is powered on and the main controller is running normally.

- At this time, all robot joint control is fully handed over to user-defined joint commands, including both legs. Use caution in this mode to prevent the robot from losing balance and falling.

- CUSTOM mode can only be entered from PREP mode or DAMP mode.

- CUSTOM mode can only switch to DAMP mode or PREP mode.

💡During secondary development, especially in CUSTOM mode, it is recommended to use a hoist to protect the robot throughout the process.

4.6 PROTECT Mode

- The robot automatically enters PROTECT mode when runtime issues occur, such as exceeding limits or falling.

- In PROTECT mode, the joint state is the same as DAMP Mode the same.

- PROTECT mode is a safe mode and is less likely to cause accidental injury.

- Body Operations

There are four buttons on the MASTER MINI back panel:

- Power button

  - Press and hold for 3 seconds to power on; press and hold again for 3 seconds to power off.

  - After powering off, wait 6 seconds before powering on again.

- WLAK： Tap to enter WALK mode.

💡 Make sure the robot has switched to PREP mode and is standing steadily on the ground before switching to WALK mode.

- STAND： Tap to enter PREP mode; the robot will fully straighten.

- F1： customized botton

- Joystick Control

💡 Note that joystick button mappings vary by software version. Please confirm the current robot software version in the "Check Version" section.

6.1Master Mini Joystick

6.1.1 Power on & off

- Power On: Press and hold the Home button for 5 seconds to power on. LED1, LED2, LED3 and LED4 all light up instantly upon startup. LED1 stays on steadily after boot completion.

- Power Off: Press and hold the Home button for 5 seconds to power off. LED1, LED2, LED3 and LED4 flash synchronously during shutdown. All indicator lights turn off after shutdown.

6.1.2 Charging

- Low battery: LED2 flashes as an alert.

- Charging in progress: LED2 keeps flashing.

- Fully charged: LED2 changes to steady on.

6.1.3 Basic Capabilities

| Actions/Functions | Buttons | Prerequisites |
| --- | --- | --- |
| Enter Master Mini Agent | L2 + R2 + A |  |
| Enter Soccer Agent | L2 + R2 + B |  |
| Enter Hi Chat Agent | L2 + R2 + X | This feature requires an internet connection |
| Enter Dance Agent | L2 + R2 + Y |  |
| Enter Lion Dance Agent | L2 + R2 + UP |  |
| Enter DAMP mode | L2 + Back |  |
| Enter PREP mode | L2 + Start |  |
| Enter WALK mode | R2 + A | Under PREP mode |
| locomotion control | Left stick | Under WALK mode |
| rotation control | Right stick | Under WALK mode |
| head rotation | Direction buttons | Under WALK mode |

6.1.4 Agent Capabilities

| Agent | Actions/Functions | Buttons | Prerequisites |
| --- | --- | --- | --- |
| Master Mini | Shake Hand | A | Under WALK mode |
|  | Wave | B | Under WALK mode |
|  | Ultraman | X | Under WALK mode |
|  | Respect | Y | Under WALK mode |
|  | Lucky Cat | R + A | Under WALK mode |
|  | Dabbing | R + B | Under WALK mode |
|  | Cheer | R + X | Under WALK mode |
|  | Carry | R + Y | Under WALK mode |
|  | CNY Dance | L + UP | Under WALK mode |
|  | Rock Dance | L + LEFT | Under WALK mode |
|  | Future Dance | L + RIGHT | Under WALK mode |
|  | Get Up | L2 + UP | Under DAMP/PREP mode |
| Soccer | Shake Hand | A | Under WALK mode |
|  | Wave | B | Under WALK mode |
|  | Get Up | L2 + UP | Under DAMP/PREP mode |
|  | Track Ball | L2 + A | Under WALK mode |
|  | Chase Ball | L2 + B | Under WALK mode |
|  | Kick Ball | L2 + X | Under WALK mode |
|  | Power Kick | L2 + Y | Under WALK mode |
|  | Siuuu | L + Y | Under WALK mode |
|  | Arms Crossed | L + B | Under WALK mode |
|  | Point to Sky | L + UP | Under WALK mode |
|  | Cradle Dance | L + DOWN | Under WALK mode |
| Dance | Rock Dance | L+A | Under WALK mode |
|  | CNY Dance | L+B | Under WALK mode |
|  | Future Dance | L+X | Under WALK mode |
|  | MJ Dance | L+Y | Under WALK mode |
|  | Arabic Dance | L+DOWN | Under WALK mode |
|  | Boxing Style Kick | L+LEFT | Under WALK mode |
|  | Roundhouse Kick | L+RIGHT | Under WALK mode |
|  | Mounts & Pals | L+R+A | Under WALK mode |
|  | Reform Breeze | L+R+B | Under WALK mode |
| Hi Chat | Default | L2+A | Under WALK mode |
|  | XiaoBao | L2+B | Under WALK mode |
|  | Gentle Ella | L2+X | Under WALK mode |
|  | Grumpy Jax | L2+Y | Under WALK mode |
|  | English Tutor | L2+UP | Under WALK mode |
|  | Cute Pet | L2+DOWN | Under WALK mode |
|  | Alien Weirdo | L2+LEFT | Under WALK mode |
|  | Say Hello | R2 + Y | Enable chat |
| Lion Dance | Dance Prepare | L+A | Under DAMP/PREP mode |
|  | Dance Walk | L+B | Under WALK mode |
|  | Lion Dance 1 | L+X | Under WALK mode |
|  | Lion Dance 2 | L+Y | Under WALK mode |

6.1.5 Xbox Joystick

Use a standard Xbox joystick.Before use, confirm that the joystick control mode is receiver mode and that the operating mode shows three lights steadily on.

6.1.6 Basic Capabilities

| Actions/Functions | Buttons | Prerequisites |
| --- | --- | --- |
| Enter Master Mini Agent | LT + RT + A |  |
| Enter Soccer Agent | LT + RT + B |  |
| Enter Hi Chat Agent | LT + RT + X |  |
| Enter Dance Agent | LT + RT + Y |  |
| Enter Lion Dance Agent | LT + RT + UP |  |
| Enter DAMP mode | LT + Back |  |
| Enter PREP mode | LT + Start |  |
| Enter WALK mode | RT + A | Under PREP mode |
| locomotion control | Left stick | Under WALK mode |
| rotation control | Right stick | Under WALK mode |
| head rotation | Direction buttons | Under WALK mode |

6.1.7 Agent Capabilities

| Agent | Actions/Functions | Buttons | Prerequisites |
| --- | --- | --- | --- |
| Master Mini | Shake Hand | A | Under WALK mode |
|  | Wave | B | Under WALK mode |
|  | Ultraman | X | Under WALK mode |
|  | Respect | Y | Under WALK mode |
|  | Lucky Cat | RB + A | Under WALK mode |
|  | Dabbing | RB + B | Under WALK mode |
|  | Cheer | RB + X | Under WALK mode |
|  | Carry | RB + Y | Under WALK mode |
|  | CNY Dance | LB + UP | Under WALK mode |
|  | Rock Dance | LB + LEFT | Under WALK mode |
|  | Future Dance | LB + RIGHT | Under WALK mode |
|  | Get Up | LT + UP | Under DAMP/PREP mode |
| Soccer | Shake Hand | A | Under WALK mode |
|  | Wave | B | Under WALK mode |
|  | Get Up | LT + UP | Under DAMP/PREP mode |
|  | Track Ball | LT + A | Under WALK mode |
|  | Chase Ball | LT + B | Under WALK mode |
|  | Kick Ball | LT + X | Under WALK mode |
|  | Power Kick | LT + Y | Under WALK mode |
|  | Siuuu | LB + Y | Under WALK mode |
|  | Arms Crossed | LB + B | Under WALK mode |
|  | Point to Sky | LB + UP | Under WALK mode |
|  | Cradle Dance | LB + DOWN | Under WALK mode |
| Dance | Rock Dance | LB+A | Under WALK mode |
|  | CNY Dance | LB+B | Under WALK mode |
|  | Future Dance | LB+X | Under WALK mode |
|  | MJ Dance | LB+Y | Under WALK mode |
|  | Arabic Dance | LB+DOWN | Under WALK mode |
|  | Boxing Style Kick | LB+LEFT | Under WALK mode |
|  | Roundhouse Kick | LB+RIGHT | Under WALK mode |
|  | Mounts & Pals | LB+RB+A | Under WALK mode |
|  | Reform Breeze | LB+RB+B | Under WALK mode |
| Hi Chat | Default | LT+A | Under WALK mode |
|  | XiaoBao | LT+B | Under WALK mode |
|  | Gentle Ella | LT+X | Under WALK mode |
|  | Grumpy Jax | LT+Y | Under WALK mode |
|  | English Tutor | LT+UP | Under WALK mode |
|  | Cute Pet | LT+DOWN | Under WALK mode |
|  | Alien Weirdo | LT+LEFT | Under WALK mode |
|  | Say Hello | RT + Y | Enable chat |
| Lion Dance | Dance Prepare | LB+A | Under DAMP/PREP mode |
|  | Dance Walk | LB+B | Under WALK mode |
|  | Lion Dance 1 | LB+X | Under WALK mode |
|  | Lion Dance 2 | LB+Y | Under WALK mode |
|  | Lion Dance 3 | LB+UP | Under WALK mode |

- Connect to Robot

- Use Agents

8.1 What Is an Agent?

An Agent is a "skill pack" installed in the robot's brain. It can also be understood as an application running on the robot. Like switching apps on a phone, switching Agents lets the robot enter the corresponding capability scenario and use different interaction methods and task capabilities.

8.2 How to Switch Agents

- Switch via the mobile App

  - Tap the Agent name in the top-left corner to switch to another Agent. Each Agent has different capabilities — for example, Soccer Master can play soccer with you, Hi Chat can talk with you using different personas, and Dance Master can perform a variety of dances from around the world.

- Switch via the Joystick

| Switch Agents | Master Mini joystick buttons | Xbox joystick buttons |
| --- | --- | --- |
| Enter Master Mini Agent | L2 + R2 + A | LT + RT + A |
| Enter Soccer Agent | L2 + R2 + B | LT + RT + B |
| Enter Hi Chat Agent | L2 + R2 + X | LT + RT + X |
| Enter Dance Agent | L2 + R2 + Y | LT + RT + Y |
| Enter Lion Dance Agent | L2 + R2 + UP | LT + RT + UP |

8.3 Official Agents

Master Mini includes 6 official Agents covering entertainment, conversation, teaching, competition, and other scenarios.

8.3.1 Master Mini

A general-purpose basic Agent that supports daily actions such as waving, handshaking, and bowing with folded hands. It can also perform several fun dances for everyday interaction scenarios.

Video1, local

Functions list

| Functions | Master Mini joystick buttons | Xbox joystick buttons | Prerequisites |
| --- | --- | --- | --- |
| Shake Hand | A | A | Under WALK mode |
| Wave | B | B | Under WALK mode |
| Ultraman | X | X | Under WALK mode |
| Respect | Y | Y | Under WALK mode |
| Lucky Cat | R + A | RB + A | Under WALK mode |
| Dabbing | R + B | RB + B | Under WALK mode |
| Cheer | R + X | RB + X | Under WALK mode |
| Carry | R + Y | RB + Y | Under WALK mode |
| CNY Dance | L + UP | LB + UP | Under WALK mode |
| Rock Dance | L + LEFT | LB + LEFT | Under WALK mode |
| Future Dance | L + RIGHT | LB + RIGHT | Under WALK mode |
| Get Up | L2 + UP | LT + UP | Under DAMP/PREP mode |

8.3.2 Soccer

Designed for soccer scenarios. It can autonomously track, chase, kick, and shoot the ball, and can accurately reproduce signature celebration moves from soccer stars such as Cristiano Ronaldo and Messi.

Video2, local

Functions list

| Functions | Master Mini joystick buttons | Xbox joystick buttons | Prerequisites |
| --- | --- | --- | --- |
| Shake Hand | A | A | Under WALK mode |
| Wave | B | B | Under WALK mode |
| Get Up | L2 + UP | LT + UP | Under DAMP/PREP mode |
| Track Ball | L2 + A | LT + A | Under WALK mode |
| Chase Ball | L2 + B | LT + B | Under WALK mode |
| Kick Ball | L2 + X | LT + X | Under WALK mode |
| Power Kick | L2 + Y | LT + Y | Under WALK mode |
| Siuuu | L + Y | LB + Y | Under WALK mode |
| Arms Crossed | L + B | LB + B | Under WALK mode |
| Point to Sky | L + UP | LB + UP | Under WALK mode |
| Cradle Dance | L + DOWN | LB + DOWN | Under WALK mode |

8.3.3 Hi Chat

An intelligent conversational Agent that can switch flexibly among fun personas such as a cute child, tutor, or pet, meeting needs for casual chat and entertaining companionship.

See details in Voice Interaction。

Video 3, local

Functions list

| Functions | Master Mini joystick buttons | Xbox joystick buttons | Prerequisites |
| --- | --- | --- | --- |
| Default | L2 + A | LT + A | Under WALK mode |
| XiaoBao | L2 + B | LT + B | Under WALK mode |
| Gentle Ella | L2 + X | LT + X | Under WALK mode |
| Grumpy Jax | L2 + Y | LT + Y | Under WALK mode |
| English Tutor | L2 + UP | LT + UP | Under WALK mode |
| Cute Pet | L2 + DOWN | LT + DOWN | Under WALK mode |
| Alien Weirdo | L2 + LEFT | LT + LEFT | Under WALK mode |
| Say Hello | R2 + Y | RT + Y | Enable chat |

8.3.4 Teach

A hands-on learning Agent. Users physically guide the robot through a motion, and the device quickly recognizes and accurately reproduces it, making custom actions easy to create.

Video 4, local

8.3.5 Dance

Specialized in a wide range of Chinese and Western dances, including folk, trendy, and classic dance moves, while also mastering advanced actions such as roundhouse kicks.

💡 Full-body dances require a dedicated venue. Rubber, cement, or tile floors are recommended; high-resistance surfaces such as carpet increase the risk of falling.

Video 5, local

Functions list

| Functions | Master Mini joystick buttons | Xbox joystick buttons | Prerequisites |
| --- | --- | --- | --- |
| Rock Dance | L + A | LB + A | Under WALK mode |
| CNY Dance | L + B | LB + B | Under WALK mode |
| Future Dance | L + X | LB + X | Under WALK mode |
| MJ Dance | L + Y | LB + Y | Under WALK mode |
| Arabic Dance | L + DOWN | LB + DOWN | Under WALK mode |
| Boxing Style Kick | L + LEFT | LB + LEFT | Under WALK mode |
| Roundhouse Kick | L + RIGHT | LB + RIGHT | Under WALK mode |
| Mounts & Pals | L + R + A | LB + RB + A | Under WALK mode |
| Reform Breeze | L + R + B | LB + RB + B | Under WALK mode |

8.3.6 Lion Dance

Focused on traditional folk performance, skillfully performing authentic Southern-style lion dance and showcasing the charm of traditional Chinese culture.

Functions list

| Functions | Master Mini joystick buttons | Xbox joystick buttons | Prerequisites |
| --- | --- | --- | --- |
| Dance Prepare | L + A | LB + A | Under DAMP/PREP mode |
| Dance Walk | L + B | LB + B | Under WALK mode |
| Lion Dance 1 | L + X | LB + X | Under WALK mode |
| Lion Dance 2 | L + Y | LB + Y | Under WALK mode |

- Service Start/Stop

9.1 Use the App

- On the control page, tap "Settings" to enter the settings page.

- On the settings page, tap "Restart Robot" to restart the robot.

9.2 Use a Terminal

- Use another device toconnect to the robot through a terminal

- Run the following commands in any directory on the robot:

💡 Note: restarting the robot service fully restarts the robot firmware. Make sure the robot is properly supported on a stand or lying flat on the ground before doing this.

- Charging

10.1 5Ah Battery Charging

- Insert the charging plug according to the indicated direction.

- The charging status is shown as follows: the green light indicates a full charge, and the red light indicates charging in progress.

10.2 2Ah Battery Charging

- Insert the charging plug according to the indicated direction.

- The charging status is shown as follows: the green light indicates a full charge, and the red light indicates charging in progress.

- Voice Interaction

12.1 Introduction

12.1.1 Hi Chat Agent

For v1.3.1.1 and above, the Master Mini robot is equipped with AI real-time voice dialogue. This feature can be enabled or disabled via the “Hi Chat” Agent in the mobile app.

12.1.1.1 Usage Instructions

- Open the “Hi Chat” entry in the mobile app, tap and select a persona. When you hear a voice prompt similar to “Hello, I am the Master Mini robot. How can I assist you?”, the real-time voice dialogue function is activated, and you can start talking to the robot.

- Once real-time voice dialogue is enabled, the robot will automatically turn its head to face the nearest person in front of it.

- AI voice interaction requires an internet connection.

12.1.1.2 Voice Commands

Note: When using a custom persona, conflicting system prompts may cause voice commands to fail.

Master Mini robots currently support voice commands that make the robot perform corresponding actions. The large model may also autonomously trigger the following actions based on the current conversation context. The action set includes:

| Action Category | Command Name | Trigger Condition |
| --- | --- | --- |
| Basic Movement | Turn Left | Triggered when explicit commands such as "turn left" or "turn to the left" are received. |
|  | Turn Right | Triggered when explicit commands such as "turn right" or "turn to the right" are received. |
|  | Turn Left One Full Circle | Triggered when commands such as "turn left one full circle" or "turn counterclockwise one full circle" are received. |
|  | Turn Right One Full Circle | Triggered when commands such as "turn right one full circle" or "turn clockwise one full circle" are received. |
|  | Walk Forward | Triggered when commands such as "walk forward" or "go forward" are received. |
|  | Walk Backward | Triggered when commands such as "walk backward" or "move back" are received. |
|  | Move Left | Triggered when commands such as "move left" or "move to the left" are received. |
|  | Move Right | Triggered when commands such as "move right" or "move to the right" are received. |
| Interaction Actions | Wave | Triggered when commands such as "wave" or "wave your hand" are received, or when expressing goodbye. |
|  | Handshake | Triggered when explicit social commands such as "shake hands" or "let's shake hands" are received. |
|  | Greet | Triggered only when the user explicitly greets the robot, such as "hello" or "Hi MASTER MINI"; the reply should include a greeting. |
|  | Nod | Triggered when the robot needs to express agreement or approval, such as responding to "the weather is nice today" or "did I do it right?"; the reply may use an affirmative tone. |
|  | Shake Head | Triggered when the user asks about unknown content, such as "what does this word mean?"; execute this action if the knowledge base has no answer. |

13. Firmware Version

13.1  Check Version

13.1.1 Check via App

After connecting to the robot, tap "Settings - About" to view the robot's current version.

13.1.2 Check via Terminal

After connecting to the robot's board, run the following command to view the current system version of the robot.

13.2 Version Upgrade

13.2.1 Before Upgrading

❗ When installing a new version of software, the robot's motion control program will be stopped, and the robot's joints will not exert force. Before upgrading, ensure the robot is in damping mode or motion control is stopped and has good support (e.g., placing it flat on the ground or using a stand to support the robot). Do not cut off power or interrupt the process during the upgrade.

- Make sure the installation package matches the current MASTER MINI model and the target version.

- Make sure the robot's battery level, network, and terminal connection are stable.

- If you have modified any configuration files, back up important configurations and logs first.

13.2.2 Upgrade via App

In the robot settings, tap "Firmware Update".

- Before installing the firmware, make sure the robot has entered damping or ready mode, and the battery level is above 50%.

13.2.3 FAQ

13.2.3.1 Slow pip Downloads

The perception process depends on some Python libraries installed via pip. If downloads are slow, you can switch the pip download source and run the upgrade again.

sudo su

pip config set global.index-url https://pypi.mirrors.ustc.edu.cn/simple

./v1.1.1.0-release-aarch64.run

13.2.3.2 apt update Fails

If the upgrade log indicates that apt update failed, you can use the fishros tool to switch sources and try the upgrade again.

# Recommended: use fishros and select in order:

# [5] One-click source change

# [1] Change system source only

# [1] Add ROS/ROS2 source

wget http://fishros.com/install -O fishros && . fishros

- Troubleshooting and Support

14.1 Restore Factory Settings

When MASTER MINI cannot return to a usable state due to configuration changes, software errors, or environment changes, you can restore factory settings to reinstall the factory software.

14.1.1 When to Use

- Abnormal behavior after modifying the robot's configuration, which cannot be recovered through normal configuration rollback.

- Technical support recommends reinstalling the factory software to rule out environment issues.

- You need to restore the robot to a software state close to its initial delivery.

14.1.2 Before You Begin

💡

Restoring factory settings may erase existing configurations and local changes. Before proceeding, back up important configurations, logs, and custom files, and make sure the robot is in a stable, well-supported state.

- Confirm that the current robot model and the recovery package are compatible with MASTER MINI.

- Make sure the power, network, and terminal connection are stable, and do not cut off power during the recovery process.

- If you are not sure whether a recovery is needed, contact technical support first.

14.1.3 Perform the Recovery

After logging in to the robot's board, navigate to the recovery directory and run the recovery program. The following command uses the installation package name in the current recovery directory; if there are multiple packages in the directory, use the file that matches your device version.

14.1.4 After the Recovery

- Wait for the recovery program to finish, and restart the robot as prompted.

- Confirm that the robot starts normally and that basic services and network connection are restored.

- Reconfigure the network, App, voice, or custom parameters that you want to keep.

- If the robot still behaves abnormally after recovery, record the symptoms and provide logs to technical support as described on the "Log Retrieval" page.

14.2 Log Retrieval

When MASTER MINI has issues, follow the steps on this page to retrieve logs and send them to technical support. Logs help locate when the issue occurred, the running state, and related error information.

14.2.1 Privacy and Purpose

Our company will, without violating any applicable laws and regulations or involving any personal privacy information, collect only data related to the operation of the robots for the purposes of fault detection, problem localization, and statistical analysis.

14.2.2 Upload Logs via App

- After connecting to the robot via the App, go to "Settings".

- Tap "Log Upload".

- Select the time range to upload, making sure it covers the time when the issue occurred.

- The upload takes about 5-10 minutes. During this process, keep the robot powered on and connected to the network. Both the App logs and the robot logs will be uploaded together.

- After the upload is complete, contact customer service to report the issue. Our technical staff will retrieve your uploaded log files through the management console.

14.2.3 Retrieve Logs via Terminal

Before you start, record the time the issue occurred, the symptoms, and the operation you were performing at the time as accurately as possible. The robot uses the UTC+8 time zone by default; if you are in a different time zone, convert the time before filling in the log time range.

- Log in to the robot via terminal.

- Confirm the time the issue occurred and prepare the start and end times for the logs. The time format is YYYYMMDD-HHMMSS.

- Run the log export command.

- Select a range that covers the time before and after the issue. For example, if the issue occurred around 20200808-120810, you can export logs about 10 seconds before and after:

- Send the log package to technical support, along with the symptoms, the time of occurrence, and the reproduction steps.

14.3 Remote Support

14.3.1 Introduction

When the robot has issues and you need remote assistance, follow these steps to get remote support from our staff.

14.3.2 Information to Send to Technical Support

- Device ID and one-time password.

- The robot's current status: whether it can power on, whether it can connect to the network, and its current mode.

- Symptoms: for example, unable to walk, abnormal App connection, upgrade failure, etc.

- The time the issue occurred and the operation you were performing at the time.

- Whether logs have been uploaded, or exported by yourself via terminal.

14.3.3 Remote Assistance Procedure

💡

You can choose from the following two remote assistance methods:

- Connect your personal computer to the robot via SSH; technical support can then connect to your computer via remote desktop and subsequently to the robot via SSH.

- Install remote desktop software on the robot; you can use a display and keyboard/mouse to open the remote desktop software, allowing support personnel to connect directly to the robot.